Athens Township Zoning Code

Subsection 6.04 (e) Fences, Walls, Terraces, Stoops, and Hedges

1. No fence, wall or hedge two (2) feet or greater in height shall be erected without the issuance                 of a Certificate of Zoning Compliance and permit fees have been paid. Applications for such   permit shall include plans and drawings showing the actual and accurate shape and dimensions of the property upon which the fence, wall or hedge is to be erected; the exact height, location, length, type of material and type of construction of such proposed fence or wall; the location of the buildings on the lot; or any such other information as deemed necessary for such permit.

Fences, walls or hedges less than two (2) feet in height do not require a Certificate of Zoning

Compliance.

2. Unless specifically noted, the provisions of this Code shall not apply to fences, terraces or    walls less than two (2) feet in height above the average natural grade, nor to terraces, steps, or unroofed stoops not over three (3) feet high above the level of the floor of the ground story.

3. The only permitted types of fences are chain link, picket, slot, rail, and solid fences.

4. A new Zoning Certificate is required whenever an existing fence is completely taken down and replaced with a new fence.

5. All fences and walls shall be structurally sound, safe, and properly finished at all times. Ground areas between fences and property lines and between fences shall be kept properly maintained at all times.

6. Fences shall be designed, constructed, and finished so the supporting members thereof shall face the property of the owner of the fence.

7. The height of a fence, wall, or hedge shall be measured from the established grade line to the highest point of the fence posts and finials. Any light fixture placed on a pier or post may not exceed a height of one (1) foot above the fence, wall, or hedge. The height may not be artificially increased by the use of mounding unless otherwise required by the zoning district regulations.

8. Front Setback area.

a. Open fences and hedges are permitted inside or along the property line of the front setback

area.

b. Any open fences or hedges shall not exceed four (4) feet in height.

c. No closed fence shall be located in the front setback area.

9. Side Setback Area.

a. Open or closed fences, walls, and hedges are permitted inside or along the property line of

a side setback area.

b. Any fence, wall or hedge shall not exceed six (6) feet in height.

10. Rear Setback Area.

a. Open or closed fences, walls, and hedges are permitted inside or along the property line of

the rear setback area.

b. Any fence, wall or hedge shall not exceed six (6) feet in height.

Page 1 of 2

11. Prohibited Uses.

a. No fence, wall or hedge or other landscaping plantings or materials shall be located within

any drainage easement, or apparent drainage course for any parcel or subdivision which

would be detrimental to the public health safety and welfare.

b. No fence, wall or hedge or other landscape plantings or materials shall be located so as to

adversely affect the vision of drivers on the public streets or from driveways intersecting

public streets.

c. No fence, wall, hedge, plantings or landscape plantings or materials shall visibly obscure,

hide, or screen fire hydrants, street address numbering, or other security or emergency

service equipment, controls or components.

12. Prohibited Materials.

a. Fences, walls or other landscaping equipped with or having barbed wire, spikes, sharp

points or any similar device or an electrical charge sufficient to cause shock are prohibited.

b. Chicken wire, poultry wire or hex netting fence consisting of a galvanized or PVC coated

material shall be prohibited except in the case of the keeping of non-domestic small

animals as permitted by conditional use in Subsection 6.04 (j).

c. This section shall not be construed or applied to prohibit underground invisible fences

installed for the purpose of confining pets to property.

Page 2 of 2